Name: __________________________ (     )	Sec 4 _____	Date: ___________

## Resolution of vectors

1.

A lamp is suspended from two wires as shown in the diagram. The tension in each wire is 4.5 N

a. Calculate the magnitude of the resultant

force exerted on the lamp by the wires.

b. What is the weight of the lamp? Explain your answer.

4.5 N                      4.5 N

40o 40o

2.

O

Two tugs A and B pull a ship a long the direction XO. Tug A exerts a force on the ship of 3.0 x 104 N at an angle of 15o to XO. Tug B pulls with a force of 1.8 x 104N at and angle  to XO.

a. Find the angle  for the resultant

force on the boat to be along XO;

(Hint: when the resultant force is along XO,

the component of the resultant force in the

direction perpendicular to XO is zero.)

b. Find the value of this resultant force.

tug B

tug A

30 kN



18 kN

15o

X

3. A body of mass 10 kg is sliding down a slope at a constant speed as

shown below. Taking g as 10 m/s2, calculate

- the component of the weight along the inclined plane, hence the force  needed to stop the object from sliding,

- the reaction force R.

R

mg

Component of the weight along the inclined plane

30o

- A microphone of mass 500 g is hung from the ceiling by a wire.

  - What is the tension in the wire?

  - A horizontally force F pulls the wire to make an angle of 30o with the vertical as shown in the figure below. Calculate the new tension in the wire.

30o

F

tension